DDTOP ULB Glass Plate Level Gauge

Reliable Isooctane Unit Level Measurement

In-Situ Display Instrument

RESULTS

- Solve the problem of strong corrosiveness of isooctane unit medium with professional design；

- Better ensure that the device can operate stably.

APPLICATION

Equipment：Isooctane Unit

Medium：Isobutane、Butene、Sulphuric Acid

Medium Characteristics：Strong Corrosion

CUSTOMER

Shandong Huabin Chemical Technology Co., Ltd.

CHALLENGE

The isooctane production process uses isobutane and butene in carbon tetra(C4) separated from catalytically cracked liquid hydrocarbons by gas rectification，The process of producing the isooctane by liquid phase reaction at a low temperature with 89%-98% sulfuric acid as a catalyst. The reaction temperature is generally between 2 ~ 18 ° C, and the design reaction temperature is generally 10 ° C.

Sulfuric acid is an important process medium, sulfides such as SO2 are used as process products in the reaction, and alkali liquid is a raw material for process reaction, which is extremely corrosive to ordinary glass plates.

SOLUTIONS

DDTOP’s ULB glass plate level gauge can solve the corrosive problems caused by the medium to the instrument:：

- Select No. 20 alloy or Hastelloy as the wetted material according to the concentration of sulfuric acid；

- For the sulphide problem, strictly comply with the requirements of NACE MR0103 in the procurement and manufacturing process；

- Designed in accordance with the welded structure in the PED certification during the design process to ensure stable operation in dangerous conditions;

- For the glass plate level gauge for measuring alkaline medium, the latest type of alkali-resistant glass on the market can be used for safe and reliable operation for more than one year without increasing the mica protection pad.

Dandong Top Electronics Instrument (Group) Co.,Ltd.

Reliable Process Instrumentation and Automation Solution Provider

Sales and Service Contact

HQ Address: No.10 Huanghai Street, Zhenxing District, Dandong, Liaoning,

China, 118000

Tel：+86-0415-6226466

Fax：+86-0415-6227341

Website: www.ddtop.com/en

Email: itrade@ddtoptrade.com